| Alumno: | García González Aarón Antonio | 11 |
| --- | --- | --- |
| Grupo: | 3CV2 |  |
| Unidad de Aprendizaje: | Arquitectura de computadoras |  |
| Profesora: | Vega García Nayeli |  |
| Practica #12: | Carta ASM |  |
| Fecha: | Domingo 24 de Mayo de 2020 |  |

# Instrucciones de practica

Implemente la etapa arquitectura para encontrar el número de 1’s en un arreglo de 9 localidades y mostrar el resultado en un display de 7 segmentos, de acuerdo con la siguiente arquitectura y lógica de control, tome en cuenta que la arquitectura debe ser implementada de manera modular.

# Unidad de control

# Código de implementación

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

entity unidad_control is

Port ( ini : in STD_LOGIC;

clr : in STD_LOGIC;

clk : in STD_LOGIC;

a0 : in STD_LOGIC;

z : in STD_LOGIC;

la : out STD_LOGIC;

ea : out STD_LOGIC;

lb : out STD_LOGIC;

ec : out STD_LOGIC;

eb : out STD_LOGIC);

end unidad_control;

architecture Behavioral of unidad_control is

type estados is (e0,e1,e2);

signal actual,siguiente : estados;

begin

process(clk, clr)

begin

if (clr = '1') then

actual <= e0;

elsif (rising_edge(clk)) then

actual <= siguiente;

end if;

end process;

process(actual,ini,z,a0)

begin

la <= '0';

lb <= '0';

ea <= '0';

eb <= '0';

ec <= '0';

case actual is

when e0 =>

lb <= '1';

if (ini = '1') then

siguiente <= e1;

else

la <= '1';

siguiente <= e0;

end if;

when e1 =>

ea <= '1';

if (z = '1') then -- arreglo igual a cero

siguiente <= e2;

else -- arreglo diferente de cero

if (a0 = '1') then

eb <= '1';

siguiente <= e1;

else

siguiente <= e1;

end if;

end if;

when e2 =>

ec <= '1';

if (ini = '1') then

siguiente <= e2;

else

siguiente <= e0;

end if;

end case;

end process;

end Behavioral;

# Código de simulación

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

entity tb_unidad_control is

end tb_unidad_control;

architecture Behavioral of tb_unidad_control is

component unidad_control

Port ( ini : in STD_LOGIC;

clr : in STD_LOGIC;

clk : in STD_LOGIC;

a0 : in STD_LOGIC;

z : in STD_LOGIC;

la : out STD_LOGIC;

ea : out STD_LOGIC;

lb : out STD_LOGIC;

ec : out STD_LOGIC;

eb : out STD_LOGIC);

end component;

-- input signs

signal ini, clk, clr, a0, z : STD_LOGIC;

-- output signs

signal la, lb, ea, eb, ec : STD_LOGIC;

-- Clock period

constant clk_period : time := 10ns;

begin

-- Instanciate

UC : unidad_control Port map (

ini => ini,

clr => clr,

clk => clk,

a0 => a0,

z => z,

la => la,

ea => ea,

lb => lb,

ec => ec,

eb => eb

);

-- Clock process

CLOCK : process

begin

clk <= '0';

wait for clk_period/2;

clk <= '1';

wait for clk_period/2;

end process;

-- process stimulli

SP : process

begin

clr <= '1';

wait for 30 ns;

clr <= '0';

wait for 60 ns;

ini <= '1';

wait for 10 ns;

ini <= '0';

wait for 50 ns;

a0 <= '1';

wait for 20 ns;

a0 <= '0';

wait for 20 ns;

a0 <= '1';

wait for 10 ns;

a0 <= '0';

wait for 120 ns;

z <= '1';

wait;

end process;

end Behavioral;

# Forma de onda de la simulación

# Diagrama RTL

# Registro

# Código de implementación

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

use IEEE.NUMERIC_STD.ALL;

entity Registro is

Port ( la : in STD_LOGIC;

ea : in STD_LOGIC;

clk : in STD_LOGIC;

clr : in STD_LOGIC;

da : in STD_LOGIC_VECTOR (8 downto 0);

qa : out STD_LOGIC_VECTOR (8 downto 0));

end Registro;

architecture Behavioral of Registro is

signal aux_a : STD_LOGIC_VECTOR(8 downto 0); -- bus

begin

process(clk,clr,la,ea)

begin

if (clr = '1') then

aux_a <= "000000000";

elsif(rising_edge(clk)) then

if (la = '0' and ea = '0') then

aux_a <= aux_a;

elsif (la = '1' and ea = '0') then

aux_a <= da;

elsif (la = '0' and ea = '1') then

aux_a <= to_stdlogicvector(to_bitvector(aux_a) SRL 1);

end  if;

end if;

end process;

qa <= aux_a;

end Behavioral;

# Código de simulación

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

entity tb_Registro is

end tb_Registro;

architecture Behavioral of tb_Registro is

component Registro is

Port ( la : in STD_LOGIC;

ea : in STD_LOGIC;

clk : in STD_LOGIC;

clr : in STD_LOGIC;

da : in STD_LOGIC_VECTOR (8 downto 0);

qa : out STD_LOGIC_VECTOR (8 downto 0));

end component;

-- input signs

signal la, ea, clk, clr : STD_LOGIC;

signal da : STD_LOGIC_VECTOR(8 downto 0);

-- output signs

signal qa : STD_LOGIC_VECTOR(8 downto 0);

-- clock period definition

constant clk_period : time := 10 ns;

begin

-- Instaciate component

REG : Registro Port map(

la => la,

ea => ea,

clk => clk,

clr => clr,

da => da,

qa => qa

);

-- clock process

CLOCK : process

begin

clk <= '0';

wait for clk_period/2;

clk <= '1';

wait for clk_period/2;

end process;

-- Stimulus process

SP : process

begin

da <= "110011001";

wait for 150 ns;

clr <= '0';

la <= '1';

ea <= '0';

wait for 150 ns;

la <= '0';

ea <= '1';

wait for 150 ns;

wait;

end process;

end Behavioral;

# Forma de onda de la simulación

# Diagrama RTL

# Contador

# Código de implementación

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

use IEEE.STD_LOGIC_ARITH.ALL;

use IEEE.STD_LOGIC_UNSIGNED.ALL;

entity Contador is

Port ( lb : in STD_LOGIC;

eb : in STD_LOGIC;

clr : in STD_LOGIC;

clk : in STD_LOGIC;

qb : out STD_LOGIC_VECTOR (3 downto 0));

end Contador;

architecture Behavioral of Contador is

constant zero: std_logic_vector(3 downto 0) := "0000";

begin

process(clk, clr, eb, lb)

variable aux_qb : STD_LOGIC_VECTOR(3 downto 0);

begin

if (clr = '1') then

aux_qb := zero;

elsif (rising_edge(clk)) then

if (lb='1' and eb='1') then

aux_qb := aux_qb;

elsif (lb='1' and eb='0') then

aux_qb := zero;

elsif (lb='0' and eb='1') then

aux_qb := aux_qb + 1;

end if;

end if;

qb <= aux_qb;

end process;

end Behavioral;

# Código de simulación

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

entity tb_Contador is

end tb_Contador;

architecture Behavioral of tb_Contador is

component Contador is

Port ( lb : in STD_LOGIC;

eb : in STD_LOGIC;

clr : in STD_LOGIC;

clk : in STD_LOGIC;

qb : out STD_LOGIC_VECTOR (3 downto 0));

end component;

-- input signs

signal lb, eb, clr, clk : STD_LOGIC;

-- output signs

signal qb : STD_LOGIC_VECTOR(3 downto 0);

begin

-- Instaciate component for test

COUNT : Contador Port map(

lb => lb,

eb => eb,

clr => clr,

clk => clk,

qb => qb

);

-- Clock process

CLOCK : process

begin

clk <= '0';

wait for 5ns;

clk <= '1';

wait for 5ns;

end process;

-- Stimulus process

SP : process

begin

clr <= '1';

lb <= '0';

eb <= '0';

wait for 150 ns;

clr <= '0';

eb <= '1';

wait for 150 ns;

eb <= '0';

lb <= '1';

wait for 150 ns;

wait;

end process;

end Behavioral;

# Forma de onda de la simulación

# Diagrama RTL

# Decodificador

# Código de implementación

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

entity Decodificador is

Port ( qb : in STD_LOGIC_VECTOR (3 downto 0);

digito_out : out STD_LOGIC_VECTOR (6 downto 0));

end Decodificador;

architecture Behavioral of Decodificador is -- "gfedcba"

constant digito_0 : STD_LOGIC_VECTOR(6 downto 0) := "0111111";

constant digito_1 : STD_LOGIC_VECTOR(6 downto 0) := "0000110";

constant digito_2 : STD_LOGIC_VECTOR(6 downto 0) := "1011011";

constant digito_3 : STD_LOGIC_VECTOR(6 downto 0) := "1001111";

constant digito_4 : STD_LOGIC_VECTOR(6 downto 0) := "1100110";

constant digito_5 : STD_LOGIC_VECTOR(6 downto 0) := "1101101";

constant digito_6 : STD_LOGIC_VECTOR(6 downto 0) := "1111101";

constant digito_7 : STD_LOGIC_VECTOR(6 downto 0) := "0000111";

constant digito_8 : STD_LOGIC_VECTOR(6 downto 0) := "1111111";

constant digito_9 : STD_LOGIC_VECTOR(6 downto 0) := "1101111";

begin

process(qb)

begin

case qb is

when "0000" => digito_out <= digito_0;

when "0001" => digito_out <= digito_1;

when "0010" => digito_out <= digito_2;

when "0011" => digito_out <= digito_3;

when "0100" => digito_out <= digito_4;

when "0101" => digito_out <= digito_5;

when "0110" => digito_out <= digito_6;

when "0111" => digito_out <= digito_7;

when "1000" => digito_out <= digito_8;

when OTHERS => digito_out <= digito_9;

end case;

end process;

end Behavioral;

# Código de simulación

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

entity tb_Decodificador is

end tb_Decodificador;

architecture Behavioral of tb_Decodificador is

component Decodificador is

Port ( qb : in STD_LOGIC_VECTOR (3 downto 0);

digito_out : out STD_LOGIC_VECTOR (6 downto 0));

end component;

-- input signal

signal qb : STD_LOGIC_VECTOR (3 downto 0);

-- output signal

signal digito_out : STD_LOGIC_VECTOR (6 downto 0);

begin

-- Insatnciate compornent

DECO : Decodificador Port map(

qb => qb,

digito_out => digito_out

);

-- Stimulus process

SP : process

begin

-- Vamos a probar con cada numero

qb <= "0000"; -- 0

wait for 10 ns;

qb <= "0001"; -- 1

wait for 10 ns;

qb <= "0010"; -- 2

wait for 10 ns;

qb <= "0011"; -- 3

wait for 10 ns;

qb <= "0100"; -- 4

wait for 10 ns;

qb <= "0101"; -- 5

wait for 10 ns;

qb <= "0110"; -- 6

wait for 10 ns;

qb <= "0111"; -- 7

wait for 10 ns;

qb <= "1000"; -- 8

wait for 10 ns;

qb <= "1001"; -- 9

wait;

end process;

end Behavioral;

# Forma de onda de la simulación

# Diagrama RTL

# Multiplexor

# Código de implementación

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

entity Multiplexor is

Port ( digito_in : in STD_LOGIC_VECTOR (6 downto 0);

ec : in STD_LOGIC;

digito_final : out STD_LOGIC_VECTOR (6 downto 0));

end Multiplexor;

architecture Behavioral of Multiplexor is

constant guion : STD_LOGIC_VECTOR(6 downto 0) := "1000000"; -- gfedcba

begin

with ec select

digito_final <= digito_in when '1',

guion when others;

end Behavioral;

# Código de simulación

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

entity tb_Multiplexor is

end tb_Multiplexor;

architecture Behavioral of tb_Multiplexor is

component Multiplexor

Port ( digito_in : in STD_LOGIC_VECTOR (6 downto 0);

ec : in STD_LOGIC;

digito_final : out STD_LOGIC_VECTOR (6 downto 0));

end component;

-- input signs

signal digito_in : STD_LOGIC_VECTOR(6 downto 0);

signal ec : STD_LOGIC;

-- output sign

signal digito_final : STD_LOGIC_VECTOR(6 downto 0);

begin

-- Instanciate component

MUX : Multiplexor Port map(

digito_in => digito_in,

ec => ec,

digito_final => digito_final

);

-- Stimulus process

SP : process

begin

ec <= '0';

wait for 20 ns;

ec <= '1';

digito_in <= "0111111"; -- 0

wait for 20 ns;

digito_in <= "0000110"; -- 1

wait for 20 ns;

digito_in <= "1011011"; -- 2

wait for 20 ns;

digito_in <= "1001111"; -- 3

wait for 20 ns;

digito_in <= "1100110"; -- 4

wait for 20 ns;

ec <= '0';

wait for 20 ns;

ec <= '1';

wait for 20 ns;

digito_in <= "1101101"; -- 5

wait for 20 ns;

digito_in <= "1111101"; -- 6

wait for 20 ns;

digito_in <= "0000111"; -- 7

wait for 20 ns;

digito_in <= "1111111"; -- 8

wait for 20 ns;

digito_in <= "1100111"; -- 9

wait for 20 ns;

ec <= '0';

wait for 20 ns;

wait;

end process;

end Behavioral;

# Forma de onda de la simulación

# Diagrama RTL

# Carta ASM (arquitectura completa)

# Código de implementación

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

entity cartaASM is

Port ( clr : in STD_LOGIC;

clk : in STD_LOGIC;

ini : in STD_LOGIC;

data_in : in STD_LOGIC_VECTOR (8 downto 0);

data_out : out STD_LOGIC_VECTOR (8 downto 0);

digit_out : out STD_LOGIC_VECTOR (6 downto 0));

end cartaASM;

architecture Behavioral of cartaASM is

-- unidad de control

component unidad_control is

Port ( ini, clr, clk, a0, z : in STD_LOGIC;

la, ea, lb, ec, eb : out STD_LOGIC);

end component;

-- registros

component Registro is

Port ( la, ea, clk, clr : in STD_LOGIC;

da : in STD_LOGIC_VECTOR (8 downto 0);

qa : out STD_LOGIC_VECTOR (8 downto 0));

end component;

-- contador

component Contador is

Port ( lb, eb, clr, clk : in STD_LOGIC;

qb : out STD_LOGIC_VECTOR (3 downto 0));

end component;

-- decodificador

component Decodificador is

Port ( qb : in STD_LOGIC_VECTOR (3 downto 0);

digito_out : out STD_LOGIC_VECTOR (6 downto 0));

end component;

-- multiplexor

component Multiplexor is

Port ( digito_in : in STD_LOGIC_VECTOR (6 downto 0);

ec : in STD_LOGIC;

digito_final : out STD_LOGIC_VECTOR (6 downto 0));

end component;

-- se�ales o buses de alambrado

signal sgnLA, sgnLB, sgnEA, sgnEB, sgnEC, sgnZ : STD_LOGIC;

signal sgnOutCounter : STD_LOGIC_VECTOR(3 downto 0);

signal sgnOutDeco : STD_LOGIC_VECTOR(6 downto 0);

signal sgnData : STD_LOGIC_VECTOR(8 downto 0);

begin

sgnZ <= '1' when sgnData = "000000000" else '0';

-- Instaciar modulo de unidad de control

UC : unidad_control

Port map(

ini => ini,

clr => clr,

clk => clk,

a0 => sgnData(0),

z => sgnZ,

la => sgnLA,

ea => sgnEA,

lb => sgnLB,

ec => sgnEC,

eb => sgnEB

);

-- Instanciar modulo de registro

REG : Registro

Port map(

la => sgnLA,

ea => sgnEA,

clk => clk,

clr => clr,

da => data_in,

qa => sgnData

);

-- Instanciar modulo de contador

CONT : Contador

Port map(

lb => sgnLB,

eb => sgnEB,

clr => clr,

clk => clk,

qb => sgnOutCounter

);

-- Instanciar modulo de decodificacion

DECO : Decodificador

Port map(

qb => sgnOutCounter,

digito_out => sgnOutDeco

);

-- Instanciar modulo de multiplexor

MUX : Multiplexor

Port map(

digito_in => sgnOutDeco,

ec => sgnEC,

digito_final => digit_out

);

data_out <= sgnData;

end Behavioral;

# Código de simulación

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

entity tb_asm is

end tb_asm;

architecture Behavioral of tb_asm is

component cartaASM

Port ( clr : in STD_LOGIC;

clk : in STD_LOGIC;

ini : in STD_LOGIC;

data_in : in STD_LOGIC_VECTOR (8 downto 0);

data_out : out STD_LOGIC_VECTOR (8 downto 0);

digit_out : out STD_LOGIC_VECTOR (6 downto 0));

end component;

-- input signs

signal ini : STD_LOGIC;

signal clk : STD_LOGIC;

signal clr : STD_LOGIC;

signal data_in : STD_LOGIC_VECTOR (8 downto 0);

-- outpuu signs

signal data_out : STD_LOGIC_VECTOR (8 downto 0);

signal digit_out : STD_LOGIC_VECTOR (6 downto 0);

constant clk_period : time := 10 ns;

begin

-- clock

CLOCK_P : process

begin

clk <= '0';

wait for clk_period/2;

clk <= '1';

wait for clk_period/2;

end process;

uut :  cartaASM Port map(

clr => clr,

clk => clk,

ini => ini,

data_in => data_in,

data_out => data_out,

digit_out => digit_out

);

-- Process stimuli

process_stimuli : process

begin

clr <= '1';

clr <= '0';

wait for 20ns;

data_in <= "101101011";

wait for 20 ns;

ini <= '1';

wait for 150 ns;

ini <= '0';

clr <= '1';

wait for 20 ns;

clr <= '0';

data_in <= "000011101";

wait for 20 ns;

ini <= '1';

wait for 150 ns;

ini <= '0';

clr <= '1';

wait for 20 ns;

clr <= '0';

data_in <= "000010000";

wait for 20 ns;

ini <= '1';

wait for 150 ns;

ini <= '0';

clr <= '1';

wait for 20 ns;

clr <= '0';

data_in <= "100001000";

wait for 20 ns;

ini <= '1';

wait for 150 ns;

ini <= '0';

clr <= '1';

wait for 20 ns;

clr <= '0';

data_in <= "000000000";

wait for 20 ns;

ini <= '1';

wait for 150 ns;

wait;

end process;

end Behavioral;

# Forma de onda de la simulación

Muestra el cambio que se hace en el data out

Muestra el corrimiento

Muestra el ejemplo de simulación a

Muestra el ejemplo de simulación b

Muestra el ejemplo de simulación c

Muestra el ejemplo de simulación d

Muestra el ejemplo de simulación e

# Diagrama RTL